# Robot Operating System

Date – 17.03.2026

Full Name – Sagnik Sahu

Introduction to ROS:

Robot Operating System, abbreviated as ROS, is an open-source operating system for developing robotic applications. It is based on the Linux operating system and offers tools and libraries for developing complex robotic applications.

ROS is based on the concept of modularity. It breaks the entire system into small programs called nodes. These nodes are connected with each other in a well-defined way. This concept of modularity has made the development and testing of complex robotic systems easier and more efficient.

ROS allows the development of robotic applications using either simulation or actual hardware. Tools like RViz and Gazebo help visualize and test robots before deployment. It also offers a number of packages for the development of robotic applications. This has made the development of robotic applications easier and faster. ROS is being used for developing a number of robotic applications.

What is ROS 2 and how does it differ from ROS 1:

ROS 2:

ROS 2 is the latest version of the Robot Operating System framework. It is designed for modern robotics applications that require reliability, scalability, and real time performance. It uses a decentralized communication system based on DDS, which allows nodes to communicate directly without a central controller. ROS 2 also provides built in security, supports multiple operating systems, and includes better tools for managing complex and distributed robot systems. It is widely used in industrial automation, autonomous vehicles, and

large-scale robotic deployments.

ROS 1:

ROS 1 is the original version of the Robot Operating System and is widely used in research and education. It follows a centralized architecture where a master node manages communication between different nodes. ROS 1 provides a large ecosystem of tools and packages, which makes it suitable for rapid development and prototyping. However, it lacks features such as real time support, strong security, and efficient handling of distributed systems, which limits its use in production environments.

Difference between ROS 1 and ROS 2:

- Architecture
ROS 1 uses a central master node to manage communication. Every node depends on this master to find and talk to others. If the master stops, the system breaks.
ROS 2 removes the master and uses DDS. Nodes discover and communicate directly. This makes the system more stable and flexible.

- Communication
ROS 1 uses its own communication methods with limited control over data flow.
ROS 2 uses DDS, which gives control over how data is sent. You set reliability, speed, and delivery rules using Quality of Service.

- Building Tool

ROS 1 uses catkin as a building tool.

ROS 2 uses Colcon as a builder tool.

- Reliability
ROS 1 has a single point of failure because of the master node.
ROS 2 avoids this problem. Even if one node fails, others keep working.

- Real time support
ROS 1 does not support strict timing tasks, so it struggles in systems where delay matters.
ROS 2 is designed for real time use. It works with real time operating systems, which is important for control systems.

- Security
ROS 1 has open communication with no built-in protection.
ROS 2 includes encryption, authentication, and access control. This is important for robots connected to networks. It uses SROS2 (DDS Security Standards)

- Performance
ROS 1 has higher delay in communication, especially in large systems.
ROS 2 is optimized for speed and handles data more efficiently, which improves response time.

- Language Support

ROS 1 only supports C++ 03 Target and have a Python Version support upto 2.7.

ROS 2 supports C++14/17 Target and supports Python 3.5+ Versions, ROS 2 Humble uses Python 3.10.

- Platform support
ROS 1 mainly works on Linux systems.
ROS 2 supports Linux, Windows, RTOS and macOS, which makes development more flexible.

What is ROS 2 Workspace:

In the ROS 2 Humble ecosystem, the development cycle begins with the establishment of a workspace—a structured directory environment where software packages are authored, managed, and compiled. Unlike the catkin workspace used in ROS 1, which relied heavily on CMake and a centralized "devel" space, the ROS 2 workspace, utilized in conjunction with the colcon build tool, adopts an "out-of-source" build philosophy that enhances modularity and prevents cross-package target name collisions.

A standard ROS 2 workspace directory, such as ros2_ws, is organized into four primary subdirectories that emerge during the build and development process.

| Directory | Technical Purpose |
| --- | --- |
| src/ | The source space where all package source code resides. This directory is the only one manually maintained by developers. |
| build/ | The intermediate storage space for build artifacts. Each package receives a dedicated subdirectory where specific build systems (e.g., CMake) are invoked. |
| install/ | The installation prefix. All executables, libraries, and setup scripts are placed here. Sourcing this directory's setup file makes the workspace's packages available to the system. |
| log/ | The logging repository. It contains historical data regarding every colcon invocation, which is essential for diagnosing build failures in complex dependency chains. |

What is ROS 2 Node:

A ROS 2 node is a basic unit of computation in a robot system. It is a small program that performs a specific task, such as reading sensor data, controlling motors, or processing information. A complete robot system is built by connecting multiple nodes that work together.

Way of communication:

- Topics
Nodes exchange continuous data using a publish and subscribe model.
Example(A camera node publishes image data, and a vision node subscribes to process those images.)

- Services
Nodes communicate using request and response.
Example(A node requests the current position of the robot, and another node returns the value.)

- Actions
Nodes handle long running tasks with feedback. Example(A navigation node sends a goal to move the robot, and receives updates until the task completes.)

Example of Nodes:

- Sensor Node: Read data from hardware sensors and publish raw data (Camera Node, Lidar Node and IMU Node)

- Perception Nodes: Process sensor data and extract useful information (Object Detection Node, SLAM Node, Face Recognition Node)

- Localization Node: Determines robot position

- Planning Node: Decides what the robot should do next (Path Planning Node and Task Planning Node)

- Control Node: Sends commands to actuators (Motor Control Node and Arm Control Node)

- Communication Node: Handles interaction with users or other systems(GUI Node and Remote-Control Node)

What are ROS 2 topics:

Topics serve as the primary medium for asynchronous, many-to-many communication in ROS 2. They are best described as a continuous data bus where nodes can exchange streams of information without direct knowledge of one another.

The topic-based communication follows a strict publisher-subscriber pattern. A node that produces data "publishes" it to a specific topic name, while one or more nodes that require that data "subscribe" to it. This decoupling allows for a highly flexible system; for example, a robot's pose can be published once and consumed simultaneously by a localization node, a logging node, and a visualization tool like RViz2.

Topics are defined by their message type, which ensures that all parties agree on the structure of the data being exchanged. In ROS 2 Humble, these types are derived from standard interface packages like geometry_msgs or sensor_msgs. For instance, a teleoperation node might publish geometry_msgs/msg/Twist messages to the /cmd_vel topic, which specifies linear and angular velocities.

What are ROS 2 Messages:

ROS 2 messages are structured data types used for communication between nodes in a ROS 2 system, typically through the publish-subscribe pattern on topics.  They enable nodes to send data (like sensor readings or control commands) without expecting a response.

Creating Custom Messages:

While standard messages cover most common robotic tasks, specialized sensors or proprietary algorithms often require custom interfaces. The creation of custom messages in ROS 2 Humble follows a rigid technical workflow to ensure compatibility across the RMW (ROS Middleware) layer.

The process for developing a custom interface involves the following steps:

- Package Setup: A new package of build type ament_cmake must be created. Pure Python packages are currently incapable of generating the required interface headers.

- Definition: Message files are placed in a msg/ directory, following a CamelCase naming convention (e.g., TargetCoordinates.msg). The file contains field types and names, such as float64 x or string status.

- Dependency Configuration: The package.xml must declare a build-tool dependency on rosidl_default_generators and an execution dependency on rosidl_default_runtime.

- CMake Integration: The CMakeLists.txt uses the rosidl_generate_interfaces() macro, specifying the message files and any dependencies, such as geometry_msgs, if the custom message embeds standard types.

Once built, these custom messages are indistinguishable from standard ROS interfaces and can be verified using the ros2 interface show command. This standardization allows for seamless integration between heterogeneous nodes; a C++ node can publish a custom message that is natively consumed by a Python node without any manual serialization effort.

What are ROS 2 Services:

While topics provide a stream of data, services facilitate a synchronous, one-to-one communication pattern based on a request and a response. This is functionally equivalent to a Remote Procedure Call (RPC) and is the preferred method for discrete tasks or state changes

| Attribute | Topics | Services |
| --- | --- | --- |
| Communication Mode | Asynchronous (Fire-and-forget) | Synchronous (Wait-for-response) |
| Topology | Many-to-Many | One-to-One |
| Typical Use Case | Continuous sensor streams (LiDAR, IMU) | Discrete commands (Reset, Trigger, Spawn) |
| Standard Profiles | Volatile, Reliable/Best-Effort | Reliable, Volatile Durability |

What is ROS 2 Launch System:

Running a complex robotic system manually would require opening dozens of terminal windows to execute each node individually. To solve this, ROS 2 provides a sophisticated "Launch" system designed to automate the execution and configuration of multiple processes simultaneously.

ros2 run vs. ros2 launch:

The CLI tools ros2 run and ros2 launch serve distinct roles in the development lifecycle.

- ros2 run <package> <executable>: Executes a single  node in the current terminal. It is primarily used for quick testing of individual modules or simple tools.

- ros2 launch <package> <launch_file>: Processes a launch description and executes all specified nodes, often across different processes. It handles parameter loading, topic remappings, and the monitoring of process exit states.

The ROS 2 launch system also introduces "Substitutions"—variables that are only evaluated at runtime. Examples include LaunchConfiguration, which retrieves command-line arguments, and FindPackageShare, which locates the installation path of a package on the filesystem. This allows for the creation of reusable launch files that can be adapted for simulation or real-world hardware by simply passing a different argument at execution time

How does ROS 2 handle node discovery without a central master (roscore):

ROS 2 handles node discovery without a central master by using DDS, where each node independently announces its presence on the network and listens for others using multicast communication. When a node starts, it shares information about the topics, services, and QoS settings it supports, and other nodes detect and match with it automatically if they are compatible. After discovery, nodes communicate directly in a peer to peer manner using unicast for data transfer, which removes the need for a central coordinator like roscore. This approach allows dynamic updates, so nodes join and leave without manual setup, and it improves fault tolerance since the system has no single point of failure.

Explain Quality of Service (QoS) in ROS 2 and why it is important for sensor data:

Quality of Service, QoS, in ROS 2 defines how data is transmitted between nodes by setting policies such as reliability, durability, history, and deadline, which control whether data must be delivered, how much data is stored, and how often messages are expected. These settings matter for sensor data because sensors generate continuous, high frequency streams where network conditions vary, so QoS helps balance speed and reliability. For example, a camera stream often uses best effort delivery to reduce delay and avoid congestion, while critical data like control commands uses reliable delivery to prevent data loss. QoS ensures stable communication, prevents overload, and allows the system to adapt to different types of data and network environments.

QoS Policies:

- Reliability: Determines whether message delivery is guaranteed (Reliable) or best-effort (Best Effort). For example, ROS 2 uses this to balance between TCP-like reliability and UDP-like efficiency.

- Durability: Controls whether a DataReader receives past samples after joining a topic. Options include Volatile (only new samples) and Transient Local (past samples are stored for late joiners).

- Deadline: Sets the maximum time interval between message publications. If a new sample isn't sent within this period, an alarm is raised (e.g., in DDS systems).

- Lifespan: Defines how long a message remains valid after being published. Messages exceeding this duration are dropped.

- Liveliness: Determines how the system detects if a publisher is still active. Options include Automatic (based on message activity) and Manual (explicit health checks).

- History & Depth: In ROS 2, "History" defines whether messages are kept last N samples or all samples, and "Depth" sets the queue size for the "keep last" mode.

- Priority (or Bandwidth): Allocates bandwidth or scheduling priority to specific traffic classes (e.g., in Cisco QoS via bandwidth or priority commands).

- Classification & Marking: Uses Class Maps and Policy Maps to identify traffic (e.g., by VLAN, DSCP, or ACL) and apply actions like marking (e.g., set dscp, set cos).

Why it is important for Sensor Data:

QoS in ROS 2 Humble is important for sensor data because sensors produce continuous, high frequency streams where network conditions often change, and QoS controls how this data is delivered. It lets you choose between reliable delivery, where no data is lost, and best effort delivery, where speed is prioritized. It also manages how many messages are stored, how long they remain available, and how often data is expected. This helps prevent delays, data loss, and network overload. For example, camera or LiDAR data often uses best effort to maintain real time performance, while critical data like control or safety signals uses reliable QoS to ensure accuracy and consistency.

Describe one real-world robotic application using ROS 2:

- A common real world application of ROS 2 is autonomous warehouse robots used in logistics centers. These robots use sensors like LiDAR and cameras to detect obstacles and navigate safely, while ROS 2 nodes handle tasks such as localization, mapping, path planning, and motion control. The system runs as multiple nodes where sensor nodes publish data, processing nodes build maps and calculate routes, and control nodes send commands to motors. ROS 2 ensures reliable and fast communication between these components, supports real time updates, and allows multiple robots to work together efficiently, which improves accuracy, reduces manual labor, and increases overall warehouse productivity.

- ROS 2 is also used in Autonomous Delivery Robots, Service Robots in Hospital and etc.

References:

- http://design.ros2.org/articles/changes.html

- Autonomous_Mobile_Robot_for_Industrial_Material_Handling_A_ROS2 (https://www.researchgate.net/publication/396955743)

- Official ROS 2 Documentation (https://docs.ros.org/en/rolling/)

- Wikipedia ROS Page (https://en.wikipedia.org/wiki/Robot_Operating_System)